The QCN program generates standard NEMA (GPRMB, GPRMC), Aviation and

special Smart Coupler data sentences.

The Smart Coupler II as an on-board EEPROM (Electrically Erasable

Programmable Read Only Memory) to hold calibration values. It is

programmed with the special data sentences produced by the QCN

program. Currently the programmable values are:

- offset voltage

- gain

- lamp dim factor

- hardware configuration

Offset Voltage

The offset voltage is programmable within a range of about +/-10mv. As it

is impossible to read what is already stored, adjust the offset as follows:

1. Set the offset value to 0

2. Read the offset with a voltmeter

3. Enter a new offset value.

4. Monitor the results, modify the value if needed. The minimum

resolution is about a millivolt, so any value within 0+/-1mv

is OK.

Gain

Just as in the old coupler, the course gain can be set to 1.25, 2.5, 5

or 10NM full scale. 1.25NM works well with the Navaid autopilot and

it is the default, so you probably won't have to do anything with

this. The QCN program calls the gain x1, x2, x4 and x8. This is

equivalent to 1.25, 2.5, 5 and 10.

Lamp Dim Factor

You can set how dim the LEDs get when the NAV lights are turned on.

There is a input line that the processor monitors for 0V (nav lights

off, LED bright) or greater than 2.5V (nav lights on, lamp dim). The

Dim Factor can be set from 1 (brightest) to 9 (dimmest). A value of 0

will defeat the dimming function completely.  In operation it is not

analog, the LED is either bright or dim.

Start the QCN program as by typing the following at the command line:

qcn [/1:2] [/P] [/N] [/F] [/?]

where    /1:2  choose serial port 1 or 2 (default:2)

/P    wait 18ms between each transmitted character

/N    insert a null character between each normal character

/F    transmit at 9600 baud using AVIATION format

/W    wait for a keypress between each transmitted character

/?    print this message

Once QCN is running, follow the menus to generate the data sentences

for course and heading.